Cells, Conductors, and Insulators

Key Concepts

- I can describe how chemical energy separates electrical charges in cells.

- I can explain how charges can flow through conductors, but not through insulators.

Batteries, Cells, and Sources

- Source

- anything that supplies electrical energy

- Cells

- a device consisting of a positive and negative electrode connected by an electrolyte that converts chemical energy into electrical energy

- when the ends (terminals) of the electrodes are connected in a pathway or circuit, charges flow through it

- a dry cell contains a moist paste as an electrolyte

- a wet cell contains a liquid electrolyte

- most household cells produce 1.5 volts of electrical energy

- Battery

- two or more cells connected together

| AA cell | 9V battery made up of 6 cells |
| --- | --- |

How A Cell Works

- because opposite charges attract each other, it takes energy to separate positive and negative charges

- in a cell, chemical reactions separate the positive and negative charges

- since work went into separating the charges, the electrons now have the energy to do other work

- ex: run a fan or a watch

- the electrical energy now stored in the cell is a form of potential energy

Coulomb

- SI unit for measuring electrical charge

- equal to the charge of 6.24x1018 electrons

Electrical Potential Differences

- a measure of the difference in electrical potential energy per unit of charge (coulomb) between the positive terminal and the negative terminal in a cell

- electrons gain electrical potential energy (a charge) when they pass through a source such as a battery

- measured in volts (V)

Conductors and Insulators

- a conductor is any material that electrical charges can move through

- electrons can move through almost all metals

- but they move through some metals more easily than others

- conductivity refers to how easily charges move through a material

- an insulator is any material through which charges cannot travel

- most electrical wiring is made out of a conductor (copper) covered with an insulator (rubber or plastic)

Assignment

Page 216 Q1-2

Page 217 Q1

Page 227 Q1,2,7

Page 272 Q 10, 11

Booklet & Below

- Why is the electrical potential difference of a source referred to as a difference?

- Explain why electrical wire are covered by an insulator.